Báo cáo Thực hành KTMT buổi 11

Họ và tên: Nguyễn Đức Phú

MSSV: 20215116

Assignment 1:

- Vẽ hình tam giác đều:

- Code:

.eqv  HEADING    0xffff8010

.eqv  MOVING     0xffff8050

.eqv  LEAVETRACK 0xffff8020

.eqv  WHEREX     0xffff8030

.eqv  WHEREY     0xffff8040

.text

main:

addi  	$a0, $zero, 90

jal   	ROTATE

jal   	GO

sleep1:

addi  	$v0,$zero,32

li    	$a0,16000

syscall

jal     	UNTRACK

goDOWN:

addi	$a0, $zero, 180

jal     	ROTATE

sleep2:

addi 	$v0,$zero,32

li      	$a0,9000

syscall

jal     	UNTRACK

jal     	TRACK

go120:

addi  	$a0, $zero, 150

jal   	ROTATE

sleep3:

addi    	$v0,$zero,32

li      	$a0,15000

syscall

jal     	UNTRACK

jal     	TRACK

goLEFT:

addi  	$a0, $zero, 270

jal   	ROTATE

sleep4:

addi    	$v0,$zero,32

li      	$a0,15000

syscall

jal	UNTRACK

jal	TRACK

go30:

addi	$a0, $zero, 30

jal	ROTATE

sleep5:

addi	$v0,$zero,32

li	$a0,15000

syscall

jal	UNTRACK

goLEFT1:

addi	$a0,$zero,270

li	$a0,10000

syscall

jal       STOP

end_main:

li $v0,10

syscall

#-----------------------------------------------------------

# GO procedure, to start running

# param[in]    none

#-----------------------------------------------------------

GO:     	li    	$at, MOVING

addi  	$k0, $zero,1    	# to  logic 1,

sb    	$k0, 0($at)     	# to start running

jr    	$ra

#-----------------------------------------------------------

# STOP procedure, to stop running

# param[in]    none

#-----------------------------------------------------------

STOP:   	li    	$at, MOVING

sb    	$zero, 0($at)  	# to stop

jr    	$ra

#-----------------------------------------------------------

# TRACK procedure, to start drawing line

# param[in]    none

#-----------------------------------------------------------

TRACK:  	li    	$at, LEAVETRACK

addi  	$k0, $zero,1

sb    	$k0, 0($at)

jr    	$ra

#-----------------------------------------------------------

# UNTRACK procedure, to stop drawing line

# param[in]    none

#-----------------------------------------------------------

UNTRACK:	li    	$at, LEAVETRACK

sb    	$zero, 0($at)

jr    	$ra

#-----------------------------------------------------------

# ROTATE procedure, to rotate the robot

# param[in]    $a0, An angle between 0 and 359

#                   0 : North (up)

# 90: East (right)

# 180: South (down)

# 270: West (left)

#-----------------------------------------------------------

ROTATE: 	li 	$at, HEADING

sw 	$a0, 0($at)

jr 	$ra

- Kết quả chạy:

- Vẽ hình vuông:

- Code:

.eqv  HEADING    0xffff8010

.eqv  MOVING     0xffff8050

.eqv  LEAVETRACK 0xffff8020

.eqv  WHEREX     0xffff8030

.eqv  WHEREY     0xffff8040

.text

main:

addi  	$a0, $zero, 90

jal   	ROTATE

jal   	GO

sleep1: 	addi  	$v0,$zero,32

li    	$a0,7500

syscall

jal     	UNTRACK

goDOWN: 	addi	$a0, $zero, 180

jal     	ROTATE

sleep2: 	addi 	$v0,$zero,32

li      	$a0,6000

syscall

jal     	UNTRACK

jal     	TRACK

goRIGHT: 	addi  	$a0, $zero, 90

jal   	ROTATE

sleep3: 	addi    	$v0,$zero,32

li      	$a0,10000

syscall

jal     	UNTRACK

jal     	TRACK

goDOWN1:	addi  	$a0, $zero, 180

jal   	ROTATE

sleep4: 	addi    	$v0,$zero,32

li      	$a0,10000

syscall

jal	UNTRACK

jal	TRACK

goLEFT:	addi	$a0, $zero, 270

jal	ROTATE

sleep5:	addi	$v0,$zero,32

li	$a0,10000

syscall

jal	UNTRACK

jal	TRACK

goUP:	addi	$a0,$zero,0

jal	ROTATE

sleep6:	addi	$v0,$zero,32

li	$a0,10000

syscall

jal     	UNTRACK

goRIGHT1: 	addi  	$a0, $zero, 90

jal   	ROTATE

sleep7:	addi	$v0,$zero,32

li	$a0,12000

syscall

jal       STOP

end_main:

li        $v0,10

syscall

#-----------------------------------------------------------

# GO procedure, to start running

# param[in]    none

#-----------------------------------------------------------

GO:     	li    	$at, MOVING

addi  	$k0, $zero,1    	# to  logic 1,

sb    	$k0, 0($at)     	# to start running

jr    	$ra

#-----------------------------------------------------------

# STOP procedure, to stop running

# param[in]    none

#-----------------------------------------------------------

STOP:   	li    	$at, MOVING

sb    	$zero, 0($at)  	# to stop

jr    	$ra

#-----------------------------------------------------------

# TRACK procedure, to start drawing line

# param[in]    none

#-----------------------------------------------------------

TRACK:  	li    	$at, LEAVETRACK

addi  	$k0, $zero,1

sb    	$k0, 0($at)

jr    	$ra

#-----------------------------------------------------------

# UNTRACK procedure, to stop drawing line

# param[in]    none

#-----------------------------------------------------------

UNTRACK:	li    	$at, LEAVETRACK

sb    	$zero, 0($at)

jr    	$ra

#-----------------------------------------------------------

# ROTATE procedure, to rotate the robot

# param[in]    $a0, An angle between 0 and 359

#                   0 : North (up)

# 90: East (right)

# 180: South (down)

# 270: West (left)

#-----------------------------------------------------------

ROTATE: 	li 	$at, HEADING

sw 	$a0, 0($at)

jr 	$ra

- Kết quả chạy:

- Vẽ ngôi sao 5 cánh:

- Code:

.eqv  HEADING    0xffff8010

.eqv  MOVING     0xffff8050

.eqv  LEAVETRACK 0xffff8020

.eqv  WHEREX     0xffff8030

.eqv  WHEREY     0xffff8040

.text

main:

addi  	$a0, $zero, 90

jal   	ROTATE

jal   	GO

sleep1: 	addi  	$v0,$zero,32

li    	$a0,15000

syscall

jal     	UNTRACK

goDOWN: 	addi	$a0, $zero, 180

jal     	ROTATE

sleep2: 	addi 	$v0,$zero,32

li      	$a0,8000

syscall

jal     	UNTRACK

jal     	TRACK

go120: 	addi  	$a0, $zero, 162

jal   	ROTATE

sleep3: 	addi    	$v0,$zero,32

li      	$a0,6000

syscall

jal     	UNTRACK

jal     	TRACK

go30:	addi  	$a0, $zero, 306

jal   	ROTATE

sleep4: 	addi    	$v0,$zero,32

li      	$a0,6000

syscall

jal	UNTRACK

jal	TRACK

goleft:	addi	$a0, $zero, 90

jal	ROTATE

sleep5:	addi	$v0,$zero,32

li	$a0,6000

syscall

jal	UNTRACK

jal	TRACK

goL:	addi	$a0,$zero,234

jal	ROTATE

li	$a0,6000

syscall

jal	UNTRACK

jal	TRACK

go1:	addi	$a0,$zero,18

jal	ROTATE

li	$a0,6000

syscall

jal	UNTRACK

go12:	addi	$a0,$zero,90

jal	ROTATE

li	$a0,6000

syscall

jal       STOP

end_main:

li        $v0,10

syscall

#-----------------------------------------------------------

# GO procedure, to start running

# param[in]    none

#-----------------------------------------------------------

GO:     	li    	$at, MOVING

addi  	$k0, $zero,1    	# to  logic 1,

sb    	$k0, 0($at)     	# to start running

jr    	$ra

#-----------------------------------------------------------

# STOP procedure, to stop running

# param[in]    none

#-----------------------------------------------------------

STOP:   	li    	$at, MOVING

sb    	$zero, 0($at)  	# to stop

jr    	$ra

#-----------------------------------------------------------

# TRACK procedure, to start drawing line

# param[in]    none

#-----------------------------------------------------------

TRACK:  	li    	$at, LEAVETRACK

addi  	$k0, $zero,1

sb    	$k0, 0($at)

jr    	$ra

#-----------------------------------------------------------

# UNTRACK procedure, to stop drawing line

# param[in]    none

#-----------------------------------------------------------

UNTRACK:	li    	$at, LEAVETRACK

sb    	$zero, 0($at)

jr    	$ra

#-----------------------------------------------------------

# ROTATE procedure, to rotate the robot

# param[in]    $a0, An angle between 0 and 359

#                   0 : North (up)

# 90: East (right)

# 180: South (down)

# 270: West (left)

#-----------------------------------------------------------

ROTATE: 	li 	$at, HEADING

sw 	$a0, 0($at)

jr 	$ra

- Kết quả chạy:

Assignment 2:

- Code:

.eqv KEY_CODE   0xFFFF0004

.eqv KEY_READY  0xFFFF0000

.eqv DISPLAY_CODE   0xFFFF000C

.eqv DISPLAY_READY  0xFFFF0008

.text

li   $k0,  KEY_CODE

li   $k1,  KEY_READY

li   $s0, DISPLAY_CODE

li   $s1, DISPLAY_READY

loop:   	nop

WaitForKey:

lw   $t1, 0($k1)

beq  $t1, $zero, WaitForKey

ReadKey:

lw   $t0, 0($k0)

WaitForDis:

lw   $t2, 0($s1)

beq  $t2, $zero, WaitForDis

Kiemtra:

KiemTraE:

beq	$t3, 1, KiemTraX

beq	$t0, 101, Co

KiemTraX:

beq	$t3, 2, KiemTraI

beq	$t0, 120, Co

KiemTraI:

beq	$t3, 3, KiemTraT

beq	$t0, 105, Co

KiemTraT:

beq	$t3, 4, Encrypt2

beq	$t0, 116, Co

Encrypt:

addi	$t3, $zero, 0

Encrypt2:

ChuHoa:	bgt 	$t0, 90, ChuThuong

blt	$t0, 65, ChuThuong

addi	$t0, $t0, 32

j	ShowKey

ChuThuong:

bgt 	$t0, 122, ChuSo

blt	$t0, 97, ChuSo

addi	$t0, $t0, -32

j	ShowKey

ChuSo:	bgt 	$t0, 57, Khac

blt	$t0, 48, Khac

addi	$t0, $t0, 0

j	ShowKey

Khac:

addi 	$t0, $zero, 42

ShowKey:

sw 	$t0, 0($s0)

nop

beq	$t3, 4, Exit

j loop

Co:	addi	$t3, $t3, 1

j	Encrypt2

Exit:

li	$v0, 10

syscall

- Kết quả chạy thử:

Assignment 3:

- Code:

.eqv	HEADING	0xffff8010

.eqv  	MOVING	0xffff8050

.eqv  	LEAVETRACK 	0xffff8020

.eqv  	WHEREX     	0xffff8030

.eqv 	WHEREY     	0xffff8040

.eqv KEY_CODE   	0xFFFF0004

.eqv KEY_READY  	0xFFFF0000

.eqv DISPLAY_CODE   	0xFFFF000C

.eqv DISPLAY_READY  	0xFFFF0008

.text

main:

li   	$s2,  KEY_CODE

li   	$s3,  KEY_READY

li   	$s0, DISPLAY_CODE

li  	$s1, DISPLAY_READY

start:

WaitForKey_start:

lw  	$t1, 0($s3)

beq  	$t1, $zero, WaitForKey_start

ReadKey_start:

lw   	$t0, 0($s2)

beq	$t0, 32, loop	# dau space thi bat dau

j start	#neu khong thi lap lai

loop:

WaitForKey:

lw  	$t1, 0($s3)

beq  	$t1, $zero, WaitForKey

ReadKey:

lw   	$t0, 0($s2)

WaitForDis:

lw   	$t2, 0($s1)

beq  	$t2, $zero, WaitForDis

Input:

InputW:	beq 	$t0, 119, Len

beq	$t0, 87, Len

InputS:	beq 	$t0, 115, Xuong

beq	$t0, 83, Xuong

InputA:	beq 	$t0, 97, Trai

beq	$t0, 65, Trai

InputD:	beq 	$t0, 100, Phai

beq	$t0, 68, Phai

j	InputExit

Len:

addi    $a0, $zero, 0

jal     ROTATE

jal     GO

jal     UNTRACK

jal     TRACK

addi    $v0,$zero,32

li      $a0,100

syscall

j	ShowKey

Xuong:

addi    $a0, $zero, 180

jal     ROTATE

jal     GO

jal     UNTRACK

jal     TRACK

addi    $v0,$zero,32

li      $a0,100

syscall

j	ShowKey

Trai:

addi    $a0, $zero, 270

jal     ROTATE

jal     GO

jal     UNTRACK

jal     TRACK

addi    $v0,$zero,32

li      $a0,100

syscall

j	ShowKey

Phai:

addi    $a0, $zero, 90

jal     ROTATE

jal     GO

jal     UNTRACK

jal     TRACK

addi    $v0,$zero,32

li      $a0,100

syscall

j	ShowKey

InputExit:

beq	$t0, 32, end_main

j	ShowKey

ShowKey:

sw 	$t0, 0($s0)

nop

j 	loop

end_main:

jal     	UNTRACK

jal 	STOP

li	$v0, 10

syscall

GO:

li    $at, MOVING

addi  $k0, $zero,1

sb    $k0, 0($at)

jr    $ra

ROTATE:

li    $at, HEADING

sw    $a0, 0($at)

jr    $ra

STOP:

li    $at, MOVING

sb    $zero, 0($at)

jr    $ra

TRACK:

li    $at, LEAVETRACK

addi  $k0, $zero,1

sb    $k0, 0($at)

jr    $ra

UNTRACK:

li    $at, LEAVETRACK

sb    $zero, 0($at)

jr    $ra

- Kết quả chạy thử: